STUDENT NAME

REGISTRATION NUMBER

Designing a power quality improvement conditioner using neural networks

### Abstract

One of the major concerns in electricity industry today is power quality. It becomes  especially important  with the introduction of advanced and complicated devices, whose performance is very sensitive  to the quality of power supply. The electronic devices are very sensitive to disturbances and thus industrial loads become less tolerant to power quality problems such as voltage dips, voltage sags, voltage flickers,  harmonics and load unbalance etc. At present, a wide range of very flexible controllers, which capitalize on newly available power electronics components, are emerging for custom power applications. Among these, the distribution static compensator, dynamic voltage restorer and unified power quality conditioner which is based on the VSC principle are used for power quality improvement.

### Introduction

Power quality is the set of limits of electrical properties that allows electrical system to function in proper manner without significant loss of performance Like flexible ac transmission system, the  term  custom power use for distribution system. Just as facts improve the reliability and quality of power transmission system, the custom power enhances the quality and reliability of power that is delivered to customers. The main causes of a poor power quality are harmonic currents, poor power  factor,  supply voltage variations,  etc. In recent years the demand for the quality of electric power has been increased rapidly. Power quality problems have received a great attention nowadays because of their impacts on both utilities and customers. Voltage sag, swell, momentary interruption, under voltages, over voltages,  noise  and  harmonics  are  the most common power quality disturbances. There are many custom power devices. The devices either connected in shunt or in series or a combination of both. The devices include D-STATCOM,  DVR  and  UPQC etc. One of the most common power quality problems today is voltage dips. A voltage dip is a short time event during which a reduction in R.M.S voltage magnitude occurs. Despite a short duration, a small deviation from the nominal voltage can result in serious disturbances. A voltage dip is  caused by a fault in  the utility system, a fault within the customer’s facility or a large increase of the load current, like starting a motor or transformer energizing. Unified power quality conditioner (UPQC) is one  of  the  best  custom power device used to compensate both source and load side problems. It consists of shunt and series converters connected back to back to a common dc link. It can perform the functions of both DSTATCOM  and DVR.

### Upqc System with Control Methods

UPQC mainly includes three parts: the series active power filters, shunt active power filters and energy storage capacitors. The series and shunt active power filter couples together through the DC-link energy storage capacitors. Series APF connected to the grid and load by coupling transformer is mainly used to  adjust the load voltage amplitude and compensate the power supply voltage sag in the controlled voltage source mode. Shunt active filter connected to the load is used to compensate load currents.

  - Voltage Sag Energy

The voltage sag energy is defined as

2

E	 T 

V (t ) 

VS	0

# 1 



 dt

V

nom 

(2.1)

Where V is the magnitude of the voltage and Vnom is the nominal voltage and T is duration of the sag.

### Artificial Neural Networks

Artificial Neural Networks are relatively electronic models based on the neural structure of the brain The brain basically learns from experiences. It is natural proof that are beyond the scope of current computers    are indeed solvable by small energy efficient packages. This brain modeling also promises a less technical way to develop machine solutions. The ANN is made up of interconnecting artificial neurons.  It  is  essentially a cluster of suitably interconnected nonlinear elements of very simple form that  possess  the ability to learn. A one-layer network with R input elements and S neurons follow. In this network, each element of the input vector p is connected to each neuron input through  the  weight  matrix W.  The ith  neuron has a summer that gathers its weighted inputs and bias to form its  own  scalar output  n(i). The  various n(i) taken together form an S-element net input vector n.

  - Training an artificial neural network

Once a network has been structured for a particular application, that network is ready to be trained. To start this process the initial weights are chosen randomly. Then, the training, or learning, begins. There are two approaches to training – ‘SUPERVISED’ and ‘UNSUPERVISED’. Supervised training involves  a mechanism of providing the network with the desired output either by manually “grading” the network’s performance or by providing the desired outputs with the inputs. Unsupervised training  is  where  the  network has to make sense of the inputs without outside help. The vast bulk of networks utilize supervised training. Unsupervised training is used to perform  Some initial  Characterization  on inputs. Training can  also be classified on basis of how the training pairs are presented to the  network.  They  are ‘INCREMENTAL TRAINING’ and ‘BATCH TRAINING’. In incremental training the weights and  biases  of the network are updated each time an input is presented to the network. In batch training the weights and biases are only updated after all of the inputs have been presented.

### Design of Upqc Using Matlab Simulation

To verify the operating performance of the proposed UPQC, a 3-phase electrical system, a neural network controller with reference signal generation method is designed for UPQC and compared its performance

with Artificial neural network based controller is simulated using MATLAB software.

  - Neural network design

The aim of the control scheme is to maintain constant voltage magnitude at a point where a fault  is connected. The controller input is an error  signal obtained from the reference voltage and the value rms of  the terminal voltage is measured. Such error is processed  by neural network based controllers where the output is the angle δ, which is provided to the PWM signal.

  - Voltage control

The sinusoidal signal Vcontrol (REFERENCE VOLTAGE) is phase-modulated by means of the angle i.e, VA = Sin (ωt +δ)

VB = Sin (ωt +δ-2π/3)	(5.1)

VC = Sin (ωt +δ+2π/3)

The Vcontrol is compared against a triangular signal in order to generate the switching signal for the VSC valves.

# Vcontrol = 1p.u

Ma =	Vtri	(5.2)

### Neural Network Training Algorithm

NEWFF (PR, [S1 S2 … SN1], {TF1 TF2 … TFN1}, BTF, BLF, PF) takes,

PR – Rx2 matrix of mind and max values for input elements Si – Size of iTh layer, for N1 layers

TFi – Transfer function of iTh layer, default = ‘tansig’

BTF – Back prop network training function, default = ‘trainlm’

BLF – Back prop weight/bias learning function, default = ‘learngdm’ net = init (net);

Before training a feed forward network, the weights and biases must be initialized. The initial weights and biases are created with the command init. This function takes network object  as  input  and  returns  a network object with all weights and biases initialized. Here is all weights and biases initialized

  - Training the input

There are two types of training procedures  according to the way in which the inputs are applied applied to  the network. They are ‘incremental training’ where each training pair will be applied one after the other and ‘batch training’ in which entire set of training pairs will be applied at once. The syntaxes for them are as below

net = train (net, p, t);

  - Simulation

The function  sim simulates a network. Sim takes  the network p, and the Network objects net, and returns   the network outputs ‘k’.

K = sim (net, p);

  - Reference Signal Generation

Reference voltage generation which is used for series converter control and Reference current generation  used for shunt converter control are generated using Parks transformation. The source current is given as:

is = ia *e^j(θ)+ ib* e j^(θ -120) + ic* e ^j(θ+120)

= ia( cos(θ) + j sin(θ)) + ib ( cos(θ -120)+j sin(θ -120) +ic(cos(θ+120)+ jsin (θ +120)	(6.1)

Id = ia cos (θ) +ib cos (θ -120) + ic  cos (θ+120)	(6.2)

Iq = ia sin (θ) +ib sin (θ-120) + ic sin (θ +120) (6.3)

### Conclusion

This thesis work is mainly devoted to the study of Power Quality problems and its compensation  with Unified power quality conditioner (UPQC). Results obtained from this study provide useful information regarding the behaviour of different controllers used for power quality improvement connected  to  distribution line. The controllers mainly used for power quality improvement are Neural network controller and Artificial neural network based controller. Neural network controller with reference signal generation method is designed for Unified power quality conditioner (UPQC) and compared  its  performance  with  artificial  neural network based controller. A New functionality is added to the UPQC system to quickly extract the reference signals directly for load current and supply voltage with a minimal amount of mathematical operands. The highly developed graphic facilities available in MATLAB/SIMULINK were used to  conduct all aspects of model implementation and to carry our extensive simulation studies on test system. The simulation results show that the UPQC with neural network controller Compensates 75% of voltage sag during fault condition. While UPQC with artificial neural network based  controller compensates 95% of voltage  sag. Hence as compared to the response obtained with Fuzzy controller, Neural network based  controller  have great advantage of flexibility.

  - Scope for future work

Proposed model of UPQC is to compensate source side and also load side problems using neural network and ANN controllers. The work can be extended to compensate total drop in the system using combined NEURO-FUZZY control (Adaptive neuro fuzzy controller).

### References

Vadirajacharya G. Kinhal, Promod Agarwal, and Hari Oam Gupta (2011), Senior Member, IEEE “Performance Investigation of Neural-Network- Based Unified Power-Quality Conditioner”  ieee  transactions on power delivery, vol.26.

Ahmet Teke, Lütfü Saribulut, and Mehmet Tümay (2011) “A Novel Reference  Signal  Generation Method  for Power- Quality Improvement of Unified Power-Quality Conditioner” ieee transactions on  power  delivery.

A. Zouidi, F. Fnaiech, and K. AL-Haddad (2006), “Neural network controlled three-phase three-wire shunt active power filter,” in Proc. IEEE ISIE, Montreal, QC, Canada, Jul. 9–12,pp. 5–10.

L. H. Tey, P. L. So, and Y. C. Chu (2004), “Unified power quality conditioner for improving power quality using ANN with hysteresis control,” in Proc. Int. Conf. Power System Technology, pp. 1441–1446.

Ruiye Liu, Ning Xia, Xiaonan Wang” The Research on Fuzzy-PID Control in Unified Power Quality Conditioner”.

Timothy Ross J. (1995), “Neural network with Engineering Applications”, McGraw-Hillbook Company; University of Mexico.

HUANG Min, ZHA Xiao-ming, CHEN Yun-ping (2002), The controller of fuzzy changing structure of parallel power quality conditioner[J]. Power System Technology,26(7):11-14.

L .Dinesh born in the year 1989 is currently doing his Post Graduate course in Power Systems And

Automation in Electrical and Electronics Engineering, department of GITAM Institute of Technology,  GITAM University, Visakhapatnam, India, the B.Tech degree in  Electrical  and  Electronics  Engineering from JNTU Kakinada, Andhra Pradesh, India, in 2010.

S. Srinivasa Rao received his master degree in Electrical Power Systems from JNTU Anantapur, in 2008, B.Tech degree in Electrical and Electronics Engineering from G.M.R Institute  of  Technology,  India,  in 2004. He is currently working as an Assistant Professor of Electrical and  Electronics  Engineering  department at GITAM Institute of Technology, GITAM University, Visakhapatnam, Andhra Pradesh, India. His research interest includes Power Quality.

N. Siva Mallikarjuna Rao received his master degree in Control Systems from JNTU Anantapur, in 2006, B.Tech degree in Electrical and Electronics Engineering from Narasraopeta Engineering College, India, in 2003. He is currently working as an Assistant Professor of Electrical and  Electronics  Engineering  department at GITAM Institute of Technology, GITAM University, Hyderabad, Andhra Pradesh, India. His research interest includes Control systems.

Figure 1. Topology of upqc

Figure 2. Single layer feed forward  network

Figure 3. Matlab simulation model  of upqc

Figure 4. Voltage controller using fuzzy

Figure 5. Voltage controller using neural networks

Figure 6. Modulator signal generation

Figure 7. Membership figures for input and output

Figure 8. fuzzy interface system

Figure 9. fuzzy Rule Viewer and Fuzzy Rule Editor

### Time

Figure 10. Source voltage due to 3 phase fault

### Time

Figure 11. Load current due to 3 phase fault

Figure 12.Matlab/Simulink Model for Reference current signal generation

### Time

Figure 13. The injected voltage to the PCC at disturbance (Vinj)

### Time

Figure 14.Load voltage with neural network controller

### Time

Figure 15. Load voltage with artificial neural network

### Time

Figure 16. Load current with neural network controller

### Time

Figure 17.Load current with artificial neural network Table 1.Circuit Parameters For UPQC

| System quantity | Supply voltage(Vs) Frequency | 380Vrms 50Hz |
| --- | --- | --- |
| Series converter | Filter inductor(Lf) Filter capacitor(Cf) | 8mH 36Uf |
| Shunt converter | DC link capacitor Reference voltage Smoothing inductor | 1100uF 650Vdc 15mH |
| Non liner load (R+jwL) | 40 ohm +j 10 ohm |  |

Table 2.Comparison Of Voltage Profile Between Fuzzy Logic and  Neural Networks Controllers

| Controller | Voltage Profile (V) | Energy Loss( J ) |
| --- | --- | --- |
| Neural network controller | 0.2pu to 0.8pu and current waveform is sinusoidal | Decreased from 0.099 to 1.27* 10^(-3) |
| Neural network | 0.2pu to 0.95pu current waveform is sinusoidal | Decreased from 0.099 8.2*10^(-5) |

From the above table it is concluded that the voltage profile is increased from 0.2 to 0.8pu using Fuzzy    logic controller and 0.2 to 0.95 using neural network. Hence ANN controller provides better voltage profile which is the main requirement in the power system operation when compared with neural network controller.